PCB Design Guide -- SpinalIMU Carrier Board

ESP32-S3 Feather  +  TCA9548A I2C Mux  +  4x MPU-6050 IMU Connectors

Target: EasyEDA -> JLCPCB fabrication  |  2-layer FR4  |  ~60 x 55 mm  |  No soldering this weekend -- design now, order Mon/Tue, solder when received

1. PCB concept and design philosophy

The PCB is a "carrier board" -- it provides a stable, permanent mounting for the ESP32-S3 Feather module and the TCA9548A multiplexer chip, and exposes four labelled connectors for the IMU modules. The Ganglion board is NOT on this PCB -- it is worn separately and connects to the laptop over BLE as it does now. The PCB replaces the breadboard, eliminating loose wires, contact resistance, and the hardware issues you have been having.

Key decisions:

- Feather accepts via female headers (you do not need to unsolder it -- just drop it in)

- TCA9548A is placed as a bare SOIC-28 IC on the PCB (EasyEDA has this in its library)

- 4x right-angle 4-pin male headers on the PCB edge for IMU cables (VCC / GND / SDA / SCL)

- 100nF decoupling capacitors on each power supply pin (eliminates I2C noise that caused your dropouts)

- 4.7kohm I2C pull-up resistors on the master SDA/SCL lines

2. Bill of Materials (BOM)

All SMD passives (caps, resistors) are 0402. If you are not comfortable with 0402, use 0603 -- just search the larger size in EasyEDA. The JLCPCB part numbers below are examples; search by value/package if they are out of stock.

| Component / Description | Qty | Footprint / Package | Value | JLCPCB Part # (example) | Notes |
| --- | --- | --- | --- | --- | --- |
| Adafruit Feather ESP32-S3 (No PSRAM) | 1 | Module (via headers) | -- | Not on JLCPCB -- buy separately | Plugs into female headers on PCB |
| Female 16-pin header, 2.54mm | 2 | THT, 2.54mm pitch | -- | C124375 | Receives Feather left + right sides |
| TCA9548A I2C multiplexer IC | 1 | SOIC-28 (7.5mm body) | -- | C130026 | I2C address 0x70 (A0/A1/A2 to GND) |
| 100nF ceramic capacitor, 0402 | 6 | SMD 0402 | 100nF / 10V | C49678 | 1x per TCA VCC, 1x per IMU connector (4x) |
| 4.7kohm resistor, 0402 | 2 | SMD 0402 | 4.7k | C25900 | Pull-up on SDA (GPIO3) and SCL (GPIO4) |
| 4-pin male header, 2.54mm (right angle) | 4 | THT, 2.54mm pitch | -- | C429954 | IMU cable connectors (Pelvis, L3, T12, T4) |
| JST-PH 2-pin connector (2mm) | 1 | THT, 2mm pitch | -- | C131337 | Optional LiPo battery connector |
| LED, green, 0402 | 1 | SMD 0402 | Green | C72043 | Power indicator (optional) |
| F330ohm resistor, 0402 | 1 | SMD 0402 | 330R | C17631 | LED current limiting resistor |

3. Complete connection list (netlist)

Use this table when wiring the schematic in EasyEDA. Every connection you need is here. Channels 4-7 of the TCA9548A (pins 17-24) are unused -- leave them unconnected.

| From (component.pin) | Net name | To (component.pin) | Notes |
| --- | --- | --- | --- |
| Feather.3V3 | +3V3 | TCA9548A.VCC (pin 8) | Main 3.3V supply from Feather LDO |
| Feather.3V3 | +3V3 | IMU_CON[1-4].VCC (pin 1) | All 4 IMU connectors share 3V3 |
| Feather.GND | GND | TCA9548A.GND (pins 4,25-28) | Ground to mux |
| Feather.GND | GND | IMU_CON[1-4].GND (pin 2) | Ground to all IMU connectors |
| Feather.SDA (GPIO3) | SDA_MAIN | TCA9548A.SDA (pin 6) | Master I2C data |
| Feather.SCL (GPIO4) | SCL_MAIN | TCA9548A.SCL (pin 7) | Master I2C clock |
| TCA9548A.A0 (pin 1) | GND | GND plane | Address bit 0 = 0 -> addr 0x70 |
| TCA9548A.A1 (pin 2) | GND | GND plane | Address bit 1 = 0 |
| TCA9548A.A2 (pin 3) | GND | GND plane | Address bit 2 = 0 |
| TCA9548A.~RESET (pin 5) | +3V3 | +3V3 plane | Active low -- pull high to enable |
| TCA9548A.SC0 (pin 9) | SCL_CH0 | IMU_CON_0.SCL (pin 4) | Pelvis IMU clock |
| TCA9548A.SD0 (pin 10) | SDA_CH0 | IMU_CON_0.SDA (pin 3) | Pelvis IMU data |
| TCA9548A.SC1 (pin 11) | SCL_CH1 | IMU_CON_1.SCL (pin 4) | L3 IMU clock |
| TCA9548A.SD1 (pin 12) | SDA_CH1 | IMU_CON_1.SDA (pin 3) | L3 IMU data |
| TCA9548A.SC2 (pin 13) | SCL_CH2 | IMU_CON_2.SCL (pin 4) | T12 IMU clock |
| TCA9548A.SD2 (pin 14) | SDA_CH2 | IMU_CON_2.SDA (pin 3) | T12 IMU data |
| TCA9548A.SC3 (pin 15) | SCL_CH3 | IMU_CON_3.SCL (pin 4) | T4 IMU clock |
| TCA9548A.SD3 (pin 16) | SDA_CH3 | IMU_CON_3.SDA (pin 3) | T4 IMU data |
| R_SDA.pin1 | SDA_MAIN | Feather.SDA | 4.7k pull-up to +3V3 |
| R_SCL.pin1 | SCL_MAIN | Feather.SCL | 4.7k pull-up to +3V3 |
| C_TCA.pins | +3V3 / GND | Near TCA9548A VCC pin | 100nF decoupling |
| C_IMU[0-3].pins | +3V3 / GND | Near each IMU connector VCC | 100nF decoupling x4 |

4. EasyEDA step-by-step build guide

Follow these steps in order. Do not start PCB layout until the schematic ERC passes with zero errors.

Step 1: Create account and new project

- Go to easyeda.com -> Sign up (free). Create a new project called "SpinalIMU_PCB".

- Click "New Schematic" to open the schematic editor.

Step 2: Place the Feather ESP32-S3 symbol

- Press "P" to open Place -> Component. Search "Adafruit Feather" or place a generic 28-pin module.

- If not found: use a 2x28 connector block (2x16 + 2x12 headers). Label each pin with the Feather net name.

- Key pins: SDA=GPIO3, SCL=GPIO4, 3V3, GND, and the header rows.

Step 3: Place TCA9548A

- In Component search: type "TCA9548A". Select the SOIC-28 package version.

- Place on schematic. Rotate if needed (press R). All 28 pins will appear.

- Connect A0, A1, A2 (pins 1-3) to GND net. Connect ~RESET (pin 5) to +3V3.

- Connect VCC (pin 8) to +3V3. Connect GND pins (4, 25-28) to GND.

Step 4: Add I2C master connections

- Draw wire from Feather SDA (GPIO3) to TCA9548A SDA (pin 6). Label net "SDA_MAIN".

- Draw wire from Feather SCL (GPIO4) to TCA9548A SCL (pin 7). Label net "SCL_MAIN".

- Place 2x 4.7kohm resistors (R1, R2). Connect R1 between SDA_MAIN and +3V3. R2 between SCL_MAIN and +3V3.

Step 5: Add IMU connectors (x4)

- Place 4x 4-pin connectors (search "4P connector 2.54mm" or "PinHeader 4").

- Label them IMU0_PELVIS, IMU1_L3, IMU2_T12, IMU3_T4.

- Each connector: Pin1=VCC (+3V3), Pin2=GND, Pin3=SDA (SDx), Pin4=SCL (SCx).

- Connect IMU0 SDA to TCA SD0 (pin 10), SCL to SC0 (pin 9).

- Connect IMU1 SDA to SD1 (pin 12), SCL to SC1 (pin 11).

- Connect IMU2 SDA to SD2 (pin 14), SCL to SC2 (pin 13).

- Connect IMU3 SDA to SD3 (pin 16), SCL to SC3 (pin 15).

Step 6: Add decoupling capacitors

- Place 5x 100nF capacitors. One near TCA9548A VCC (pin 8), one near each IMU connector VCC.

- Connect each cap between +3V3 and GND. Keep them physically close to the IC in the PCB layout step.

Step 7: Add power symbols and nets

- Press "P" -> Power Port. Add "+3V3" and "GND" power symbols throughout schematic.

- Verify all power connections are consistent. Run ERC (Tools -> ERC) -- fix all errors before continuing.

Step 8: Convert to PCB and do layout

- Click "Design -> Convert Schematic to PCB". A PCB window opens with all components unplaced.

- Set board outline: draw a rectangle ~60mm x 55mm (enough for Feather + TCA + connectors).

- Place Feather header footprints first (they define the PCB size). Place TCA9548A centrally.

- Place IMU connectors on the PCB edge (so cables can exit cleanly). Place decoupling caps next to TCA VCC.

- Route: run Tools -> Auto Router for a first pass. Then manually fix any crossing traces.

- Add GND copper pour on bottom layer: Place -> Copper Area -> select GND net.

Step 9: Run DRC and export for JLCPCB

- Design -> DRC. Fix all errors (unconnected nets, clearance violations).

- Fabrication -> One-click to order on JLCPCB. This pre-loads your Gerbers into the JLCPCB order form.

- Settings: 2-layer, 1.6mm FR4, HASL finish, any colour. Quantity: 5 (minimum).

- Total cost ~$2-5 USD for 5 boards + ~$15-20 shipping. Delivery 5-7 working days.

5. Adafruit Feather ESP32-S3 pinout -- key pins

The Feather has two 16-pin headers. The pins you need are on the RIGHT header (looking at the board from the top with USB-C at the top):

| Position (right header) | GPIO | Net / Function |
| --- | --- | --- |
| Pin 13 (from top) | GPIO 3 | SDA_MAIN -- connect to TCA9548A pin 6 |
| Pin 12 (from top) | GPIO 4 | SCL_MAIN -- connect to TCA9548A pin 7 |
| Pin 2 (from top) | 3V3 | +3V3 power supply for TCA + IMUs |
| Pins 1 + 16 | GND | Ground (both GND pins, connect both) |

Note: the Feather left header provides RST, additional GND, A0-A5, and SPI pins -- these are not needed for the IMU system.

6. PCB layout tips (critical for signal integrity)

- Place decoupling caps as close as physically possible to TCA9548A VCC pin (pin 8). "Close" means < 1mm trace from cap to IC pin.

- Keep SDA and SCL traces short and away from power traces. Route them as a matched pair.

- Place IMU connectors on the board edge so cables exit cleanly without folding over the board.

- Use a ground copper pour on the bottom layer. Connect all GND pads to it with vias.

- Minimum trace width: 0.2mm for signal, 0.5mm for power (+3V3, GND). EasyEDA default is 0.25mm -- fine for signals.

- Set clearance: 0.2mm minimum (JLCPCB standard). EasyEDA will warn if you violate this in DRC.

7. PCB bring-up procedure (when boards arrive)

Do this in order -- do not power on before visual inspection.

- 1. Visual inspection: check all solder joints under magnification. No bridges, no cold joints.

- 2. Continuity check (multimeter): +3V3 not shorted to GND. TCA9548A VCC to GND = high resistance.

- 3. Power on without Feather: measure 3.3V rail with multimeter at a header pin. Should read 3.3V.

- 4. Insert Feather. Power on. Measure 3.3V at each IMU connector VCC pin (pin 1).

- 5. Flash imu_reader.ino (GPIO 3/4). Connect one IMU to channel 0 (Pelvis connector). Open Serial Monitor. Confirm data rows appear at 100Hz.

- 6. Connect remaining IMUs one by one. Confirm all 4 channels stream independently.

- 7. Run record_imu_serial.py. Confirm CSV saves. Compare signal quality to breadboard.

8. Common mistakes to avoid

- Do NOT use Wire.begin(8,9) in firmware -- must be Wire.begin(3,4)

- TCA9548A SOIC-28 is small -- use flux and solder wick to fix bridges. Solder pad by pad, not all at once.

- AD0 on MPU-6050 GY-521 modules is already pulled to GND on the module -- no extra wiring needed for address 0x68.

- Do NOT connect TCA channels 4-7 to anything. Leave SC4-SC7, SD4-SD7 (pins 17-24) unconnected.

- Order at least 5 PCBs -- the minimum at JLCPCB -- in case one board has a defect.